MAINTENANCE & SAFETY CHECKLIST

2005 Nissan Armada   •   Check the box. Write the reading. Initial the line.

VEHICLE

| Date |  | Miles |  | Inspector |  |
| --- | --- | --- | --- | --- | --- |
| VIN |  | 2WD / 4WD |  | Tow pkg | Y / N |
| Visit type | Oil-change short check / 15k inspection / 30k powertrain / Baseline 270k |

Pass = normal for mileage. Fail = repair or recheck before return to service. Write the actual reading when a blank is shown.

# 1.  FLUIDS  (every visit)

☐   Engine oil level & condition    Color / level  ____________     qt added  ______

☐   Engine oil leak check    Valve covers / oil cooler O-ring / pan / front cover

☐   Coolant reservoir level & color    ________________________     cap seated  Y / N

☐   ATF — dipstick HOT    Level in range  Y / N     Color: red-amber / brown / pink / milky     Smell: ATF / burnt / sweet

☐   Power steering fluid    Level  ________     Color  ________

☐   Brake fluid    Level  ________     Color clear-amber / dark     Reservoir cap sealed

☐   Washer fluid

☐   Transfer case seep (4WD)    Wetness at seals  Y / N

☐   Front differential seep (4WD)    Y / N

☐   Rear differential seep    Y / N     pinion seal  Y / N

# 2.  ENGINE BAY

☐   Cold-start noise — timing cover    Quiet / short rattle / ongoing rattle     seconds  ______

☐   Cold-start noise — exhaust manifolds    Quiet / tick L bank / tick R bank / both     soot at flange  Y / N

☐   Idle quality    Smooth / rough     CEL  On / Off

☐   Serpentine belt    Cracks / glaze / fray / OK     tensioner play  Y / N

☐   Radiator tanks, seams, hoses    Soft / swollen / seeping  Y / N

☐   ATF cooler lines at radiator    Still connected to radiator  Y / N     wet fittings  Y / N     bypass already done  Y / N

☐   Air filter    Clean / dirty / replace     cabin filter due  Y / N

☐   Battery    Rest V ______     running V ______     terminals clean  Y / N     load test  Pass / Fail

☐   Ground straps    Block-to-chassis and body  tight / corroded

☐   PCV hose / vacuum lines    OK / cracked / oily

☐   Scan tool    Stored codes ______________     pending ______________     ATF temp ______

# 3.  TRANSMISSION ROAD CHECK

Circle the result. Do cold and again after a 15-minute drive.

| Check | Cold | Hot | Notes |
| --- | --- | --- | --- |
| Park → Reverse engagement | Good / Delay / Bang | Good / Delay / Bang |  |
| Reverse → Drive | Good / Delay / Bang | Good / Delay / Bang |  |
| 1–2 upshift | Clean / Flare / Harsh | Clean / Flare / Harsh |  |
| 2–3 upshift | Clean / Flare / Harsh | Clean / Flare / Harsh |  |
| 3–4 and 4–5 | Clean / Miss / Hunt | Clean / Miss / Hunt |  |
| TCC lock ~45–60 mph light throttle | Smooth / Shudder / Slip | Smooth / Shudder / Slip |  |
| Kickdown 5–4 / 4–3 | Clean / Late / Harsh | Clean / Late / Harsh |  |
| 4WD engage / disengage (if eq.) | Works / Binds / No | Works / Binds / No |  |

☐   ATF reject condition    STOP if pink, milky, or sweet-smelling. Do not continue the drive.

# 4.  BRAKES & ROLLING GEAR

☐   Pad thickness    LF ____ mm    RF ____ mm    LR ____ mm    RR ____ mm

☐   Rotors    Thickness / rust lip / pulse on stop     F ______     R ______

☐   Brake hoses & lines    Cracks / bulge / rust     wet caliper  Y / N

☐   Master cylinder / booster    Seepage  Y / N     pedal firm  Y / N

☐   Pedal height engine running    Spec ≥ 3.5 in @ 110 lb     measured  ________

☐   Parking brake    Spec 3–4 clicks @ 44 lb     clicks  ______     holds on grade  Y / N

☐   ABS / SLIP / VDC lamps    Prove-out then off  /  stay on  /  intermittent

☐   Tires tread    LF ____    RF ____    LR ____    RR ____    spare ____

☐   Tire age (DOT week/year)    LF ______  RF ______  LR ______  RR ______  spare ______

☐   Tire wear pattern    Even / inner shoulder / outer / cupping / center

☐   Pressures including spare    LF __  RF __  LR __  RR __  spare __

☐   Lug torque after rotation

☐   Wheel bearings / hubs    Growl / play  LF RF LR RR

☐   Alignment feel    Pull L / Pull R / Wander / Straight

# 5.  STEERING, SUSPENSION, DRIVELINE

☐   Steering play at idle

☐   Inner / outer tie rods

☐   Upper & lower ball joints / UCAs

☐   Sway-bar bushings & end links

☐   Spring perches / shackles / body mounts

☐   Shock / strut leaks

☐   Rack boots / PSF high-pressure hose

☐   Front & rear drive shafts / U-joints / slip yoke (4WD)

☐   CV / pinion / axle seals

☐   Engine and transmission mounts

# 6.  UNDERBODY / FUEL / EXHAUST / FRAME

☐   Oil pan and drain plug

☐   Transmission pan and cooler fittings

☐   Fuel tank, straps, lines, EVAP area

☐   Exhaust hangers, cats, muffler, leaks

☐   Frame rust, spare-tire carrier, running-board mounts

☐   Brake and fuel line rust

# 7.  CABIN & SAFETY

☐   Airbag lamp proves out and goes off

☐   Seatbelts latch and retract — all rows

☐   Doors / rear hatch / rear glass latch

☐   Horn

☐   Headlights, high beams, fogs, tails, brake, reverse, plate, hazards

☐   Wipers / washers front and rear

☐   Defroster and HVAC blower

☐   Mirrors / glass cracks

☐   Jack, lug wrench, spare present and usable

☐   Open recalls checked by VIN this visit    Y / N     printout attached  Y / N

# 8.  FINAL ROAD TEST

☐   Cold start — chain and manifold noise recorded

☐   No overheat after 15 min mixed driving     gauge stable  Y / N

☐   Brakes from 30 and from 60 — no pull, no pulse

☐   No new vibration 45–70 mph

☐   Shift quality matches Section 3

☐   No new warning lamps

# 9.  RESULT

| Overall | Pass / Pass with notes / Do not drive / Schedule repairs |
| --- | --- |
| Fail items |  |
| Next service type | Oil change @ ________ mi 15k @ ________ mi 30k powertrain @ ________ mi |
| Sign-off | Name ________________________ Date __________ |

Companion document: 2005 Armada Strategy & How-To Guide. Confirm specs in FSM SM5E-1T60U1.